MONTGOMERY  COUNTY  PLANNING  DEPARTMENT THE MARYLAND-NATIONAL CAPITAL PARK AND PLANNING COMMISSION

December 12, 2007

# MEMORANDUM

TO:		Historic Preservation Commission

FROM:	Clare Lise Kelly, Historic Preservation Planner

Montgomery County Planning Department

SUBJECT:	Evaluation of Amendment to the Master Plan for Historic Preservation: Individual Historic Resources

## BACKGROUND

In an ongoing effort to evaluate historic resources in Montgomery County, staff initiates amendments to the Master Plan for Historic Preservation.  Many of these amendments are created in coordination with the Planning Department’s updates of area master plans.  The Germantown area resources in this Amendment are being reviewed because the Germantown Master Plan is being updated. This Amendment also includes resources being evaluated at the request of property owners and community groups.  Two of the resources under review (#18/44 and 19/40—see list below) are already identified on the Locational Atlas and Index of Historic Sites.  The other five resources in this Amendment are nominations by property owners and interested parties.  Under the HPC’s approved Executive Regulations, the Historic Preservation Commission must review submitted nominations to the Locational Atlas and forward a recommendation to the Planning Board.

The enclosed Amendment summarizes the Atlas resources and nominations.  Also enclosed are Maryland Historical Trust inventory forms, photographs, and maps for each resource.

Following the public meeting and worksession, the HPC will take a vote on which of these resources are recommended for designation on the Master Plan for Historic Preservation, and which are recommended for removal from the Locational Atlas, or, for those properties not currently on the Atlas, resources not to be designated on the Master Plan for Historic Preservation.   The HPC recommendations will be transmitted to the Planning Board in the form of a revised Preliminary Draft Amendment.  The Planning Board will hold public hearing(s) and worksessions and transmit their recommendations to the County Council.  The County Council makes the final decision on designation of historic sites.

STAFF RECOMMENDATIONS

Designate on the Master Plan for Historic Preservation:

- Little Seneca Viaduct, B&O Railroad, Germantown, #18/44, Locational Atlas resource

- B&O Culverts and Abandoned Railroad Bed, Germantown, #19/40, Locational Atlas resource

- Burton Log House, 15107 Birmingham Drive, Burtonsville, #34-14, Nomination to Locational Atlas

- Susan B. Chase House, 4717 Drummond Avenue, Chevy Chase, #35-160, Nomination to Locational Atlas

- Seymour Krieger House, 6739 Brigadoon Drive, Bethesda, #35-161, Nomination to Locational Atlas

Do not designate on the Master Plan for Historic Preservation.

- Germantown Presbyterian Church, 19431 Walter Johnson Road, Germantown,

#19-13-8, Nomination to Locational Atlas

- Watson House, 9206 Watson Road, Silver Spring, #36-55, Nomination to Locational Atlas

##### DISCUSSION

### RESOURCES RECOMMENDED FOR DESIGNATION

- The Baltimore and Ohio Railroad’s Metropolitan Branch was important to the history of Montgomery County in ways that include economic development, suburban growth, and tourism.  This line, which runs from Washington, DC, to Point of Rocks, Maryland, became the main route west for trains coming from Washington and Baltimore.  There are four main periods of history for the railroad: the founding (1853-73); the early years (1873-90); the age of improvement (1890-1927); and modernization (1927-50).

- The Metropolitan Railroad was founded in 1853 and went bankrupt during the Civil War.  The B&O Railroad took over the building of the line, using surveys made by the original company.  Construction of this railroad line was begun in 1865 and completed in 1873.  The Metropolitan Branch was recognized as a major engineering feat of its time, with structures large and small considered excellent in design and construction.

- Two resources present rare opportunities to preserve the original single tracked railroad bed and engineering features.  These two sections of abandoned track may well be the only remaining undisturbed sections of the rail line in Montgomery County.

- The Stone Culverts and Abandoned Railroad Bed, Resource #19/40, represent the first era of the Metropolitan Branch (1853-1873).  The Metropolitan Branch was originally a single track rail line.  This section of the line was abandoned in 1927 when the line was straightened and double-tracked.   Because it was no longer in use, the culverts and single rail width bed have been preserved.  The engineer is believed to have been James A. Boyd, the contractor for the first section of the line, who set up camp where the eponymous town was later established.

- The resource is located immediately south of Harvest Glen Way.  The railroad bed and culverts were added to the Locational Atlas in 2000, at the request of the Germantown Historical Society. The Planning Board had approved a 114-unit townhouse development, Autumn Glen, located between the existing CSX tracks and the abandoned B&O tracks.   In April 26, 2000, the HPC reviewed this nomination and voted unanimously to add the resource #19/40 to the Locational Atlas.   The Planning Board agreed with the HPC and placed the resource on the Locational Atlas.

- The developer entered into an agreement with the Planning Board which included the installation of interpretive markers and preservation of the railroad bed for a walking trail.  As part of this agreement, the north culvert was removed for the construction of a stormwater management pond, and recreated re-using some of the original Black Rock stones.   This structure no longer functions as a culvert but is a display of stones banked into the railroad bed and representing the original culvert. Additional stones from the original north culvert are in storage at Black Hill Regional Park.  Some of these stones will be used as a historical display at the new Darnestown Heritage Park.   The south culvert is approximately 20 feet long, constructed of handhewn granite and Black Rock stone.

- This resource, including the railroad bed and both culverts, is on land owned and maintained by the Autumn Glen Homeowners Association.  In accordance with the Planning Board agreement, the AGHA has established a walking trail, with historic markers located at each culvert.

- Staff recommends this resource be designated on the Master Plan for Historic Preservation.  When it recommended placement on the Locational Atlas in 2000, the HPC found that the resource meets criteria 1a, 1d, and 2a.  Staff concurs.   The recommended setting is the parcel of land on which the resource is located.

The Little Seneca Viaduct, Resource #18/44, was identified on the original Locational Atlas of 1976.  This resource includes three main elements that represent the first three periods of history for the railroad.  The elements are: an abandoned railroad bed dating from c1865; stone rip-rap wall from the foot of the original 1872 wood trestle viaduct that traversed Little Seneca Creek; and the remains of the metal bridge viaduct that replaced the wooden one in 1896.  This resource is located approximately midway between Boyds and Germantown.

The abandoned single-track railroad bed, cut into the hillside, represents the first era of the Metropolitan Branch (1853-1873).  The double tracking of the Metropolitan Branch was begun in 1890.  The Germantown section was the last to be double tracked.  Because of the rough terrain in the Germantown area, the curved and single track remained in use until 1927.    Because it was no longer in use, the single rail-width bed has been preserved.  The railroad bed leads from Wisteria Drive to the Little Seneca dam.  It is currently being used as an access road to service the dam.  The road is closed to the public. While the railroad bed section in Resource #19/40 runs along relatively flat land, this section of the old railroad line had to be cut into a hillside.

A wooden trestle viaduct was built over Little Seneca Creek in the early 1870s.  The only visible remains of that earlier bridge are the stone retaining walls—which the B&O referred to as ‘riprap’—at the base of the fill on the east bank of the river.

A metal viaduct replaced the wooden trestle bridge in 1896.  Both of these bridges were single-tracked and built on a sharp (4 degree) curve which added to the difficulties in their construction.   The metal viaduct served until 1928, when the line was re-located and double-tracked.  During its period of service, it was on one of the major rail lines into the Nation’s Capital, and it became the B&O’s main line from Baltimore to the west.

The viaduct remains consist of one large stone end abutment and four stone piers.  The abutment is approximately 31 feet wide and 12 feet long.  The structure stands some 12 feet above grade at its exposed end. There are two dates chiseled in the abutment, “6.17.96” and “10.13.96”.  These dates probably represent the periods of construction of the stonework.  The newspaper accounts indicate the bridge was constructed during the period June-December 1896.

The 1896 viaduct was designed by John E. Greiner (1859-1942) who was the B&O’s Engineer of Bridges at the time.  Greiner was an accomplished civil engineer of some note—after he left the B&O in 1908, the consulting firm he founded was involved in a number of important projects,  including the Pennsylvania Turnpike, the Susquehanna River bridge at Havre de Grace, and many others.  Greiner was also probably responsible for the upgrading of the Bollman Truss spans and their replacement on the Monocacy Viaduct in the 1890-1906 period.  The bridge Greiner designed was a relatively standard design early steel viaduct, though its sharp curvature was not so common.

In 1980, a dam was constructed for a reservoir, creating Little Seneca Lake.  The dam was built in the center of the viaduct site that had been surveyed in the 1970s.  The construction of the dam removed the western section of the viaduct site, including the west stone abutment and stone piers on what was the west slope of Little Seneca Creek.

In an interagency agreement made June 22, 1982, M-NCPPC owns and operates the access road, while WSSC maintains and operates the dam.  The abandoned railroad bed is owned by M-NCPPC, and the viaduct site and rip-rap wall, located adjacent to the dam, are located on WSSC property.

Measurements taken in 1979 of the stone remains and historic photographs of the bridge indicated that it was built on a four-degree curve with a total span of about 480 feet.  It was about 105-feet above water level at its midpoint.  According to railroad historian Carlos Avery, the bridge was a relatively standard-design viaduct, with the track supported by deck-type girder spans supported by nine bents, arranged into four towers and one stand-alone bent.  There was a three-foot walkway on either side of the single track, with water barrels positioned at irregular intervals.  The tower spans were 30-feet long, with 60-foot spans between the main towers and end spans of 70 feet (at the east end) and 50 feet (at the west end).  The maximum width of the towers at the base was approximately 60 feet.

- There are three other viaducts on the Metropolitan Branch in Montgomery County.  The earliest is the 1896 Rock Creek Viaduct, Master Plan site #31/5/1, a stone arch bridge with earth fill.  The other two date from 1906: the Waring Viaduct, MP site #19/10, and Little Monocacy Viaduct, Atlas resource #12/19.  The Waring Viaduct is a three-arch stone bridge over Great Seneca Creek, and the Little Monocacy is its larger twin.

- The Little Seneca Viaduct is a rare surviving example of the original single-tracked railroad bed and engineering features of a wooden trestle bridge and metal viaduct.  It is one of the last remaining undisturbed sections of the rail line in Montgomery County, and represents the engineering accomplishments of the B&O Railroad.

This resource, including the viaduct, rip-rap wall, and railroad bed, meets criteria 1a, 1d, and 2a.  The recommended environmental setting includes stone end abutments, stone piers, stone retaining wall, and the railroad bed, now used as an access road from Wisteria Drive to the Little Seneca Lake dam.

- The Burton Log House, Resource #34-14, at 15107 Birmingham Drive, Burtonsville, has been nominated for designation by its owner, Kenneth Lourie.  Though the Burton House was not identified on the original Locational Atlas, the resource was identified as a significant structure in a 1976 survey by architectural historian Orin Bullock.  The resource escaped further notice by the preservation community until 1996 when PAC Spero investigated it for an Inter County Connector study.   This study found that the Burton House is eligible for National Register designation.

- The Burton House is significant architecturally as an early Maryland house and historically for representing the settlement era of the Burtonsville community.  The house is situated near the headwaters of Little Paint Branch, along the Prince George’s County boundary.    The west log section was a one-room structure, 16’ x 21’, with a loft above.  The east section is post and beam construction, likely dating to c1850, containing a box staircase built into the north side of the chimney.

- Tax records indicate the house was built c1800.  Hence the original log house was built before the Columbia Turnpike Company constructed a road from Georgetown to Ellicott’s Mills (1810).  The Burton family settled here in 1817, first Basil and then his son Isaac Burton, Sr. owning the land. Isaac Sr, who acquired the property in 1825, is considered the founder of Burtonsville.  By 1850, when the east wing was built, a crossroads community known as Burtons was established at the intersection of the Columbia Turnpike and the Laurel-Sandy Spring Road.   In 1851, Isaac Sr. conveyed 51¾ acres of his property, including this dwelling, to his son Isaac Jr.  Maps of 1865 and 1879 show Isaac Burton residing here, likely referring to Isaac Jr.  The property remained in the family until 1947 when it was sold by Isaac Jr.’s daughter Mary Hearn.

- Though it is not known if Isaac Sr. ever lived in this house, its historical association with him and his son, together with the rare survival of Burtonsville resources combine to make this a significant resource to the history of the Burtonsville community.  Burtonsville remained small and rural after the Civil War.  It had a population of about 50 people, with resources including a Baptist church, blacksmith shop, schoolhouse, and a store.

The Burton House has had some alterations.  Most windows are casement and fixed sash that appear to date from the early 20th century.  Dormer windows were constructed on the original log section, likely when the loft was made into a second story.  The front porch has a concrete deck.  An early east addition has a stone foundation.  A more recent south addition has concrete block foundation and chimney. Nonetheless, the resource retains the character-defining features of early Montgomery County houses, including the steeply pitched roof, log construction, and brick chimney. The resource meets criteria 1a, 1d, and 2a.   The recommended environmental setting is Lot 1, Block D; being 2.06 acres.  A wood frame stable built c1895 is a contributing resource.

The Susan B. Chase House, #35-160, has been nominated by its owner for designation on the Master Plan for Historic Preservation.  Located at 4717 Drummond Avenue, the residence is located in the Drummond subdivision, immediately north of Somerset.  Built in 1903-4, the Susan B. Chase House is one of, if not the, first houses to be designed and constructed in the Drummond Subdivision.

The Drummond Subdivision was platted in 1903.  Drummond is representative of the many subdivisions in the greater Chevy Chase area that were established by independent entrepreneurs riding on the success of the Chevy Chase Land Company.  Others include Otterbourne, 1893; Norwood Heights, 1893; and Griffith’s Additions, later Martin’s Additions, 1904.   Drummond had originally been part of an extensive estate and was subdivided around the time of the development of the nearby electric rail service on the Georgetown and Rockville line.  Accessible and regular rail service allowed the upper-middle class to reside in a rural setting and still easily commute to work in the District of Columbia.

Architecturally, the Susan B. Chase House is representative of other Drummond houses built during the period of transition between the symmetrical, showy Queen Anne style of the late 19th century, and a more conservative, symmetrical Colonial Revival of the 20th century.  The residence, as the nominator has suggested, does provide an important contribution to an understanding of suburban Montgomery County residential architecture at the turn of the twentieth century.

Staff recommends that the Chase House be designated for representing the Drummond community and its architecture.  There is precedent for such a designation.  In Chevy Chase, Takoma Park, Hyattstown and Dickerson, single structures were designated as individual sites before the communities were evaluated for, and, in these cases, ultimately designated as,  historic districts.   In Otterbourne, nine individual houses were designated that represent the settlement period of this community.  These houses were built between 1894 and 1914, were found to represent the greater community.

Like the Otterbourne subdivision, in Drummond there was an initial building campaign during which a group of houses were constructed.  In Drummond this period was 1903 to 1916.  During this period, sixteen houses were built, of which the Chase House is the first or possibly second.  Staff agrees with the nominator that the Susan B Chase House is representative of the first group of houses that were constructed at Drummond.

Staff recommends designation of the Chase House, finding that it meets criteria 1a, 1d, and 2a.  There is the potential for the HPC to evaluate in the future either a Drummond Historic District, or designation of additional individual houses that represent the period of historic significance for this community.   The recommended environmental setting is Lot 36, Subdivision 22, being 12,400 square feet.

Finally, staff recommends for designation the Seymour Krieger House, 6739 Brigadoon Drive, Bethesda, #35-161.  This resource has been nominated by its owner, who is also nominating it for listing in the National Register of Historic Places.

The Seymour Krieger House was constructed in 1958 to the designs of internationally renowned architect Marcel Breuer. The structure is the only single-family dwelling designed by Marcel Breuer in Montgomery County, and is one of four residential buildings he designed in Maryland.  The residence was built for Seymour Krieger, a communications lawyer, and his wife Rita.  The Krieger family continued to live here until 1964.

The resource is an outstanding example of the International Style.  Every aspect of the architectural and landscape designs of the Seymour Krieger House has a specific function, an ideal promulgated by the International Style. The triangular-shaped corner lot was landscaped by prolific landscape designer Dan Kiley.  The project was the first of five collaborations between Breuer and Kiley nationwide, and was one of only two projects these distinguished designers worked together on in Maryland.

The Seymour Krieger House is a landmark in the area. It is an established and familiar visual feature due to its singular physical characteristic and landscape.   Staff notes a comparable yet obviously very different building is the Frank Lloyd Wright designed house at 7927 Deepwell Drive, Bethesda, built in 1957.  Built within a year, these two residences are within a half mile area, on the east bank of Cabin John Creek.  Like the Wright house,  Breuer’s house is only residence in the county designed by that master architect.

Staff agrees with the nominator that the resource meets 2a, 2b, 2c, and 2d.   The recommended environmental setting is Lot P19, Block E, being 32,567 square feet.

# RESOURCES RECOMMENDED FOR REMOVAL FROM LOCATIONAL ATLAS

- Staff does not recommend the following resources be designated on the Master Plan for Historic Preservation.  Staff finds that these resources have neither architectural nor historical significance.

Germantown Presbyterian Church, 19431 Walter Johnson Road, Germantown, #19-13-8, Not currently listed on Locational Atlas

This resource has been nominated by the Germantown Historical Society.  The building has been greatly altered. The research form includes historic photographs of the church.  The character-defining feature of the church is the bell tower which has been removed.  The additions of a wraparound porch and substantial side wing have further obscured what was once a recognizable vernacular church building type.  Hence it is no longer representative of a building type.  There are occasions when the historic significance may outweigh diminished integrity. Such was the case with Higgins Tavern which had suffered many changes, yet was still found eligible for designation.  Staff understands that this resource was a community building, but that may be said of all church buildings, and not all churches are necessarily eligible for designation.  This resource does not have the historic significance that would be necessary to outweigh the great loss of integrity that this resource has suffered.

Watson House, 9206 Watson Road, Silver Spring, #36-55

Not currently listed on Locational Atlas

In contrast to the Chase House, staff does not recommend designation of the Watson House.  A cursory review reveals some similarities between these structures.  They both date from the early 20th century.  They are both like many other houses in suburban Montgomery County built in this time period.  They are both built on land that went through a progression from farm to estate of an upper-class Washingtonian to suburban development.   Yet there are important distinctions between these two resources.

T he Chase House represents the larger community, being the first of several houses built and representing the larger community.  The Watson House does not represent the greater community.  It is in the proximity of two other already-designated historic sites: the Condict House (Grey Rocks) and the Wilbur House.  These resources, however, draw their significance from the pre-suburban development era of Silver Spring, while the Watson House does not represent a trend, a community, or have historical connection to the other houses, other than the fact that the Watson family bought the Wilbur House at one point and lived in it before building a new house.

The Watsons had aspirations to be developers, yet they were largely unrealized.  A year after establishing a subdivision, James Watson died.  The land was largely undeveloped until 1948.  Mary Clement Watson platted Watson’s Addition to Woodside Park in 1940 and then she died five years later.  The Watson House does not represent a larger community or development.  The Watsons were not successful developers and did not build a community of buildings.   The surrounding community of Woodside Park was developed by others.  The 1918 house is an attractive Dutch Colonial residence yet is not an outstanding, early, or unique example.    There are scores of similar Dutch Colonials in Chevy Chase, Takoma Park, and Silver Spring.  Staff finds that the house has neither historical nor architectural significance.

Enclosures:

-Amendment to the Master Plan for Historic Preservation: Individual Historic Resources

-Inventory Forms, Photographs, Maps, and Related Correspondence